**************************************************************************

*******************  ASIM CARD EMULATOR V3.1 BY THE ANDROID **************

********** THIS VERSION BY CHAVEIRO'98 SIM CARD EMULATOR V4.0 ************

*******************       PCB DESIGN BY GIULIO CESARE      ***************

**************************************************************************

For more info check:  http://chaveiro.cjb.net

WHAT'S NEW on v4.0 :

Save sim file capability.

Added suport for SMS and Abreviated phone numbers. (read and record)

Added Verify of sent data. On error user will be noticed.

Added INS=2F (status), to work with recent phones.

Added detailed runtime information and description of what's going on.

Added suport for PIN2 and PUK.

Added schematic hardware to the program exe.

Added autoload and autostart funcionality from command prompt.

Change serial port usage from DSR to DCD for compability with other hardware.

Redisigned 6fxx File view with detailed description.

Added pc speaker beep on reset.

This software allows you to connect your PC to any GSM Phone.

It uses the SIM card slot as an interface.

SIM UNDERSTANDS AS SUBSCRIBER IDENTITY MODULE.

The chip card used by the GSM SIM follows the specification ISO 7816-4.

The protocol is the asynchronous half duplex T=0 protocol with active

low reset. Convention can be direct or inverse (SIM manufacturer dependent).

ASIM emulates the SIM card FILE structure and the authentification

algoritms (A3 & A8).

With Asim you can emulate easely the Motorola Test and Clone Cards.

ASIM V3.1 IS UPDATED TO SUPPORT TEST MODE IN MOTOROLA DCS1800 PHONES.

Asim uses a script type files (*.DAT) wich contains the Sim file contents

and configuration parameters, you can create a file of this type for

every SIM card you wish emulate.

The script files are ASCII so it can be edited easely with a text editor.

Note that the lines must no exceed 255 chars in lenght.

The valid options are:

!ATR = SIM ATR

!COM = DEFINES DE COMUNICATION PORT

!BAUD= DEFINES DE BAUD SPEED

!BDLY= DELAY BETWEEN BYTES SENDED.

!PIN=  THE SIM'S PIN.

!PIN2= THE SIM'S PIN2.

!PUK=  THE SIM'S PUK.

!KEY=  SIM KI KEY FOR A3A8 ALGORITHM

*XXXX= SIM MESSAGE FOR THE XXXX FILE

**=    MORE BYTES TO THE LAST SIM MESSAGE

#XXXX= SIM FILE XXXX CONTENTS

##=    MORE BYTES ADD TO LAST FILE

With the ASIM i include two *.dat files , sim.dat and clone.dat.

You must edit those for personalice the COM PORT and BDLY.

The first is for emulating a generic sim file with Test Mode enabled.

The second contains the ATR for enter Clone Mode.

In order to start emulating a card you must first load the corresponding

script file (LOAD SIM in the File menu).

Then go to the Run Auto in the Emulator menu or Run Clone in the clone

menu.

Or from the prompt type:  ASIM filename.DAT

filename.DAT will be loaded and emulator will run.

When in CLONE mode , the transfers frames between the phone and ASIM are

stored in a FRAME buffer of 4 KBYTES, you can LOAD,SAVE and COMPARE this

buffer from disk in binary form (*.BIN).

You can also LOAD and SAVE the frame buffer contents in Motorola S19 format

for assure data integrity (it's more robust than binary files and has a

checksum in each data line).

######### ADAPTER ########

In order to use ASIM you must build the adapter described below,

you will only require some basic electronics experience.

I INCLUDE FIVE FILES:      EMUSIM.BMP

CIRCLAY.GIF

README.1ST

MAX232LY.BMP

MAX232LY.GIF

THIS FILES CONTAINS TWO PCB LAYOUTS FOR THE ASIM INTERFACE ADAPTERS.

DESIGNED BY GIULIO CESARE:  fgiova@mbox.procom.it   gcesare@geocities.com

You can produce your card adapter by making a PCB with contact areas

or use an empty phone card, which also has ISO contacts, removing the

integrated chip from the non-contact side  and use a conductive ink

to draw connections to the end of the card were you connect wires.

The following diagram describes the whole interface.

This easy level adapter with only 3 transistors was worked in almost all

PC's i have tested.

If you experiment any problem you can use instead a MAX232 level converter

and two transistors as see below.

���������������������������Ŀ

C1	VCC	Supply voltage +5 V         �   C1        �        C5   �

C2	RST	Reset (active low)          �             �             �

C3	CLK	Clock                       ��������\     �     /������Ĵ

C4	RFU     NOT USED                    �   C2   ������        C6   �

C5	GND	Ground                      �        �         �        �

C6	VPP	TIRED EVER TO +5V           ��������Ĵ         ��������Ĵ

C7	I/O	DATA INPUT OUTPUT           �   C3   �         �   C7   �

C8    RFU     NOT USDED                   �        �����������        �

��������/     �     \������Ĵ

�  C4         �        C8   �

�             �             �

�����������������������������

RS232 DB9                    VCC

���������                     �

���Ŀ

PCGND (5) ������ GND        �4k �

� 7 �

�����

������������������ ISO C7 I/O PIN (7)

/c        �

�����Ŀ   �/           �

TXD (3) ������Ĵ 10k ���Ĵ\  BC547    �

�������  b�  \e        �

�        �

GND       �

�

VCC                      �

�                       �

���Ŀ                     �

�4k �                     �

� 7 �                     �

�����                     �

RXD (2) ����Ĵc                      �

\                      �

\ �b �����Ŀ         �

BC547  /��Ĵ 10k �����������

/  �  �������

�e

GND

VCC

�

���Ŀ

�4k �                     ��������� ISO C2 RESET PIN (2)

� 7 �                     �

�����                     �

DCD (6) ����Ĵc                      �

\                      �

\ �b �����Ŀ         �

BC547  /��Ĵ 10k �����������

/  �  �������

�e

GND

INTERFACE WITH A MAX232 AND TWO TRANSISTORS:

The four condensators must be 10 uf electrolitic or tantalium type.

Note that the condensators have polarity.

VCC must be conected to the card Vcc pin (ISO C1).

GND must be conecter to the card GND pin (ISO C5) and the PC serial COM GND

(pin 5 in a DB9 conector).

�������������Ŀ

+-----------�1          16�-------o VCC

+|          +�             �

=== VCC o-||-�2          15�-------o GND (CARD & RS-232)

|           �             �

+-----------�3          14�-------o DCD (6 RS232)

+  �   MAX232    �

+---||---�4          13�-- NC

|        �             �

+--------�5          12�-- NC

+   �             �

GND o-||-�6          11�-------o RESET  (ISO C2)        I/O  (ISO C7)

�             �                                 �

(2) RXD o----�7          10���������������������������������Ĵ          VCC

�             �                                 �         ���Ŀ

(3) TXD o----�8           9�������Ŀ              VCC        �         �4k �

���������������       �               �         �         � 7 �

�             ���Ŀ       �         �����

�             �4k �       �����������Ĵ

�             � 7 �                  /c

�             �����     �����Ŀ   �/

�               �������Ĵ 10k ���Ĵ\  BC547

�              /c       �������  b�  \e

� �����Ŀ   �/                        �

�Ĵ 10k ���Ĵ\  BC547                GND

�������  b�  \e

�

GND