#############################################################################

##################### GSM SIM FILE SYSTEM BY THE ANDROID ####################

#############################################################################

NAME         ADDRESS    TIPE    DATES      LEN            DESCRIPCION

-------      ---------  ------  -------    -----        ---------------

ICC-ID      3F00:2FE2    EF      BIN       $0A       CARD SERIAL NUM

ADN         7F10:6F3A    EF      REC      100*$1C    ABREVIATED DIALING NUM

FDN         7F10:6F3B    EF      REC       10*$1C    FIXED DIALING NUM

SMS         7F10:6F3C    EF      REC       15*$B0    SHORT MESSAGE

MSISDN      7F10:6F40    EF      REC       05*$1C    MOB STA IS DIALING NUM

SMSP        7F10:6F42    EF      REC       03*$2A    SORT  MESSAGE POINTER

SMSS        7F10:6F43    EF      BIN       $02       SHORT MESSAGE STATUS

LDN         7F10:6F44    EF      CICLIC    10*$1C         DIALING NUM

EXT1        7F10:6F4A    EF      REC       05*$0D    EXTENSION 1

EXT2        7F10:6F4B    EF      REC       03*$0D    EXTENSION 2

LNG         7F20:6F05    EF      BIN       $04       LANGUAGE

IMSI        7F20:6F07    EF      BIN       $09       INTER. MOBI.SUBS.IDENT.

NOIMPLE     7F20:6F10    EF      BIN

NOIMPLE     7F20:6F16    EF      BIN

KC          7F20:6F20    EF      BIN       $09       CIPHERING KEY

PLMN SELEC  7F20:6F30    EF      BIN       $30       PLMN SELECTOR

HPLMN       7F20:6F31    EF      BIN       $01       SEARCH PERIOD

ACMMAX      7F20:6F37    EF      BIN       $03       ACOUNT MAXIMUN

SST         7F20:6F38    EF      BIN       $04       SIM SERVICE TABLE

ACM         7F20:6F39    EF      CYCL      10*$03    ACUMULATED CALL METER

CCP         7F20:6F3D    EF      REC       05*$0E    CAPABILITY CONFIG PARAM

GID1        7F20:6F3E    EF      BIN       01        GROUP IDENTIFIER LEVEL 1

GID2        7F20:6F3F    EF      BIN       01        GROUP IDENTIFIER LEVEL 2

PUCT        7F20:6F41    EF      BIN       05        PRICE PER UNIT

CBMI        7F20:6F45    EF      BIN       $14       CELL BROADCAST MESSAGE ID

BCCH        7F20:6F74    EF      BIN       $10       BROADCAST CONTROL CHANEL

ACC         7F20:6F78    EF      BIN       $02       ACCESS CONTROL CLASS

FPLMN       7F20:6F7B    EF      BIN       $0C       PLMN  PROHIBIDAS

LOCI        7F20:6F7E    EF      BIN       $0B       LOCATION INFORMATION

AD          7F20:6FAD    EF      BIN       $03       ADMINISTRATIVE DATA

PHASE       7F20:6FAE    EF      BIN       $01       PHASE IDENTIFICATION

7F10         DF      BIN

7F20         DF      BIN

NOIMPLE     7F40         DF      BIN

; ###########################################################################

; ######################## IMSI / ICCID INFO ################################

; ###########################################################################

IMSI                            ICCID               CARD SERIAL

--------------------------   -----------------------------  ---------------

08 29 41 70 67 21 00 05 06   98 43 70 02 00 00 98 03 51 F3  20000 0893015-3

08 29 41 70 01 00 26 68 47   98 43 70 10 00 00 51 88 94 F5  01000 0158849-5

08 29 41 70 01 00 26 68 78   98 43 70 10 00 00 51 88 26 F8  01000 0158862-8

----------------------------------------------------------------------------

08 29 41 10 02 00 27 67 78   98 43 65 02 91 46 08 61 83 F6  20196 4801638-6

08 29 41 10 02 00 74 49 25   98 43 65 05 92 46 58 45 30 F6  50296 4855403-6

The first 3 are for SPAIN TELEFONICA (MOVISTAR) and the last 2 are for AIRTEL.

Whith the four first bytes of IMSI the phone konws the gsm network.

The four byte determine ($70=TELEFONICA) ($10 AIRTEL).

The icc id determine the card serial number (Note the nibles are inverted) and

the country in my TELEFONICA card 89 43 70 is 98 34 70 where 34 is SPAIN and

70 TELEFONICA.

IN ALL THE CARDS THAT I HAVE SEEN THE FIRST BYTE IS 98.

I will run succesfully an Airtel SIMLOCKED phone with a Movistar simulated SIM

changing only the four byte in the IMSI ($70 to $10), and erasing the $10 in

the forbiden PLMNS file. See below for PLMNS file.

; ###########################################################################

; ########################### FORBIDEN PLMNS FILE ###########################

; ###########################################################################

70=PLMN MOVISTAR

10=PLMN AIRTEL

The FILE PLMMNS contains the codes of the forbiden plmns.

Im my AITEL    SIM card the file contains: 12 F4 70 FF FF FF FF FF FF FF FF FF

In my MOVISTAR SIM card the file contains: 12 F4 10 FF FF FF FF FF FF FF FF FF

You can observe that in an Airtel file, Movistar is forbiden and viceversa.

; ###########################################################################

; ################## SIM CARD ISO 7816-4 COMPATIBLE COMMANDS ################

; ###########################################################################

COMMAND SYNTAX: CLA INS P1 P2 P3  PROC  DATA  SW1 SW2

CLA    = COMMAND CLASS IN GSM ALWAIS A0

INS    = INSTRUCTION TYPE

P1,P2  = PARAMETERS OF INSTRUCION

P3     = NUMBER OF BYTES TO I/O TO OR FORM THE CARD

PROC   = PROCEDURE BYTE RETURNED BY THE CARD (NORMALY IS THE INS BYTE)

DATA   = DATA TO OR FROM THE CARD DEPENDING OF THE INSTRUCTION TYPE

SW1,SW2= STATUS BYTES (90 00 IS OK)

; ************** SELECT FILE  DF 7F20 ********

A0 A4 00 00 02   A4   7F 20  9F 16  (9F16... THERE'IS A MESSAGE OF 16 BYTES)

; ************* GET MESSAGE OF P3 BYTES ***************

A0 C0 00 00 16  C0

85 14 00 18 7F 20 02 00 00 FB FF 03 09 09 00 12 04 00 83 8A 83 8A 90 00

; ************* READ BINARY FROM THE CARD ************

A0 B0 00 00 04 B0 03 FF FF FF 90 00 ; READS P3=4 BYTES FROM THE CURRENT FILE

; ************* READ RECORDS *************

A0 B2 01 04 05 B2   11 22 33 44 55  90 00

READS FROM RECORD (P1=1) OF CURRENT FILE (P3=5) BYTES

; ************* WRITE BINARY TO THE CARD *******

A0 D6 00 00 04 D6 12 34 56 78 90 00 ; WRITE P3=4 BYTES TO THE CURRENT FILE

; **************** RESETS COST CALL COUNTER *********

A0 DC 00 03 03 DC 00 00 00 90 00

;**************** AUTHENTICATE SUBSCRIBER ***********

AUTHENTICATE 128 BITS:   128 BITS RAND + Ki ----> A3 ----> 32 BITS SRES

A0 88 00 00 10  88   9D B9 6A B7 12 0E 6D 09 E3 A9 A4 B1 E0 AF 71 D7   9F0C

GET SRED RESPONSE:

A0 C0 00 00 0C  C0   28 6F FA 6C 6A 0C 2A 87 47 2B 7C 00  9000

; ********* VERIFY PIN (5328)  *********

A0 20 00 01 08 20 35 33 32 38 FF FF FF FF 90 00

90 00	Normal ending

6B 00	P1 / P2 Wrong

67 08	P3 Wrong

98 08	Contradiction to PIN status

98 04	Wrong secret code

98 40	Wrong secret code - Instruction blocked !

92 0x	Update sucessful after x retries

92 40 	Update impossible (card EEPROM defective)

P2=01 ---> PIN1

P2=02 ---> PIN2

; ********* CHANGE PIN (5328) ********

A0 24 00 01 10 24 35 33 32 38 FF FF FF FF 35 33 32 38 FF FF FF FF 90 00

P2=01 ---> PIN1

P2=02 ---> PIN2

; ********* OPERATE WIWHOUT PIN (5328) *********

A0 26 00 01 08 26 35 33 32 38 FF FF FF FF 90 00

P2=01 ---> PIN1

P2=02 ---> PIN2

; ********* OPERATE WITH PIN (5328) *********

A0 28 00 01 08 28 35 33 32 38 FF FF FF FF 90 00

P2=01 ---> PIN1

P2=02 ---> PIN2

; **** UNBLOCK PIN WITH PUK ****

A0 2C 00 00 10 2C 35 38 37 36 39 39 30 38 35 33 32 38 FF FF FF FF 90 00

P2=01 ---> PIN1

P2=02 ---> PIN2

; ****** INTERPRETACION MENSAGE 7F20  22 BYTES ********

A0 A4 00 00 02 A4 7F 20 9F 16    ; SELECT DF 7F20

A0 C0 00 00 16 C0                ; GET MESAGE

85 14 00 18 7F 20 02 00 00 FB FF 03 09 09 00 12 04 00 83 8A 83 8A 90 00

1-4       NI IDEA

6-7       NOBRE FICHERO  (7F20)

8-13      NI IDEA

14        HIGH NIBLE (8=PIN1 OFF , 0=PIN1 ON)

19        LOW NIBLE PIN1 COUNTER

20        LOW NIBLE PUK1 CUNTER

21        LOW NIBLE PIN2 COUNTER

22        LOW NIBLE PUK2 COUNTER

; ############################# GSM SECURITY ################################

THE A3 & A8 ALGORITHMS ARE KEY DEPENDENT ONE WAY HASH FUNCTIONS.

THE SUBSCRIBER IDENTIFICATION IS PERFORMED WHITH THE IMSI (INTERNATIONAL MOBILE

SUBSCRIBERS IDENTIFICATION) AND THE KI (INDIVIDUAL SUBSCRIBERS KEY).

The GSM network authenticates the identity of the subscriber

through the use of a challenge-response mechanism.  A 128-bit random

number (RAND) is sent to the MS.  The MS computes the 32-bit signed

response (SRES) based on the encryption of the random number (RAND)

with the authentication algorithm (A3) using the individual subscriber

authentication key (Ki). Upon receiving the signed response (SRES) from

the subscriber, the GSM network repeats the calculation to verify the

identity of the subscriber.  Note that the individual subscriber

authentication key (Ki) is never transmitted over the radio channel.  It is

present in the subscriber's SIM, as well as the AUC, HLR, and VLR

databases. If the received SRES agrees with the calculated value, the MS has

been successfully authenticated and may continue.  If the values do not match,

the connection is terminated and an authentication failure indicated to the

MS.

The SIM contains the ciphering key generating algorithm (A8)

which is used to produce the 64-bit ciphering key (Kc).  The ciphering key

is computed by applying the same random number (RAND) used in the

authentication process to the ciphering key generating algorithm (A8) with

the individual subscriber authentication key (Ki).The ciphering key (Kc) is

used to encrypt and decrypt the data between the MS and BS.

To ensure subscriber identity confidentiality, the Temporary

Mobile Subscriber Identity (TMSI) is used.  The TMSI is sent to the

mobile station after the authentication and encryption procedures have

taken place.  The mobile station responds by confirming reception of the

TMSI.  The TMSI is valid in the location area in which it was issued.  For

communications outside the location area, the Location Area Identification

(LAI) is necessary in addition to the TMSI.

Encrypted voice and data communications between the MS and the

network is accomplished through use of the ciphering algorithm A5.

Encrypted communication  is initiated by a ciphering mode request

command from the GSM network.  Upon receipt of this command, the

mobile station begins encryption and decryption of data using the

ciphering algorithm (A5) and the ciphering key (Kc).